# SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | PRATHIBA K | Reg.No.: | 192421163 |
| Department: | B TECH IT | Date: |  |

### Code of Conduct:

I PRIYADHARSHINI K S 192511248	 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

### Signature of the student: PRIYADHARSHINI K S

## ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

DMA-Based Data Transfer Algorithm

This question asks for pseudocode that transfers data from an I/O device to memory using DMA,

handles interrupts, and minimizes CPU involvement.

Aim

To design a DMA-based data transfer algorithm in which the DMA controller transfers data

directly from an I/O device to memory and interrupts the CPU only after the transfer is completed.

Input

- I/O device

- Memory address

- Data size

Output

- Data transferred from I/O device to memory

- CPU notified through interrupt

Pseudocode

BEGIN

Initialize DMA Controller

Set DMA Source = I/O Device

Set DMA Destination = Memory

Set Transfer Size = Data Size

Enable DMA Interrupt

Start DMA Transfer

WHILE DMA Transfer is NOT Complete

CPU performs other tasks

END WHILE

IF DMA Interrupt is Received THEN

Check DMA Status

IF Transfer is Successful THEN

Display "Data Transfer Completed"

ELSE

Display "Data Transfer Failed"

END IF

END IF

Disable DMA Interrupt

END

Working

- The CPU initializes the DMA controller.

- Source is set to the I/O device and destination to memory.

- The required transfer size is specified.

- DMA transfer is started.

- The DMA controller transfers data directly to memory.

- During the transfer, the CPU performs other tasks instead of handling each data byte.

- After completion, DMA generates an interrupt.

- CPU checks the DMA status and handles the result.

Why DMA is Used?

DMA minimizes CPU involvement because the DMA controller handles the actual data

transfer. The CPU is interrupted only when the transfer requires attention, making the algorithm

efficient for large data transfers.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

Interrupt Handling System with Vectored Interrupts

This question asks for pseudocode that prioritizes vectored interrupts from multiple devices

and dispatches the appropriate service routine.

Aim

To design an interrupt handling system that identifies the interrupt source, prioritizes multiple

devices, and executes the corresponding Interrupt Service Routine (ISR).

Input

- Interrupt requests from multiple devices

- Interrupt priority

- Interrupt vector table

Output

- Appropriate ISR executed for the interrupting device

Pseudocode

BEGIN

Initialize Interrupt Vector Table

Initialize Interrupt Controller

WHILE System is Running

IF Interrupt Request is Received THEN

Identify Interrupt Source

Get Interrupt Priority

Get Interrupt Vector

Select Highest Priority Interrupt

Save Current CPU State

Dispatch to Corresponding ISR

Execute ISR

Clear Interrupt Request

Restore CPU State

Return to Previous Task

ELSE

Continue Normal Execution

END IF

END WHILE

END

Working

- The interrupt vector table is initialized with the ISR address of each device.

- When multiple devices generate interrupts, the interrupt controller identifies their sources.

- The priority of each interrupt is checked.

- The highest-priority interrupt is selected first.

- The CPU saves its current state and jumps to the corresponding ISR using the interrupt vector.

- The ISR services the device.

- The interrupt request is cleared.

- The CPU restores its previous state and continues execution.

Example

Device 1 → Interrupt Vector 1 → ISR 1

Device 2 → Interrupt Vector 2 → ISR 2

Device 3 → Interrupt Vector 3 → ISR 3

If Device 2 has highest priority:

Device 2 → Vector 2 → ISR 2 → Return

Justification

Vectored interrupt handling allows the CPU to directly locate the appropriate service routine

for each device. Prioritizing interrupts ensures that important device requests are serviced first,

while normal CPU execution continues when no interrupt is pending.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

Dynamic Selection of Memory-Mapped I/O and I/O-Mapped I/O

The question asks for pseudocode to dynamically select between memory-mapped I/O and I/O

mapped I/O based on latency and bandwidth requirements.

Aim

To select the appropriate I/O mapping technique according to the required latency and

bandwidth.

Input

- Required latency

- Required bandwidth

- Threshold values

Output

- Selected I/O mapping technique

Pseudocode

BEGIN

Read Required_Latency

Read Required_Bandwidth

IF Required_Latency is HIGH AND

Required_Bandwidth is HIGH THEN

Select Memory-Mapped I/O

Display "Memory-Mapped I/O Selected"

ELSE IF Required_Latency is LOW AND

Required_Bandwidth is LOW THEN

Select I/O-Mapped I/O

Display "I/O-Mapped I/O Selected"

ELSE

Compare Required_Latency and Required_Bandwidth

IF Required_Latency is HIGH THEN

Select Memory-Mapped I/O

ELSE

Select I/O-Mapped I/O

END IF

END IF

END

Working

- Read the required latency and bandwidth.

- If both latency and bandwidth requirements are high, select Memory-Mapped I/O.

- If both requirements are low, select I/O-Mapped I/O.

- For intermediate requirements, compare the latency requirement.

- Select the technique that best satisfies the requirement.

Decision Flow

Start

│

▼

Read Latency & Bandwidth

│

▼

┌───────────────────┐

│ High Latency AND  │

│ High Bandwidth?   │

└───────┬───────────┘

Yes │       No

▼        ▼

Memory-Mapped  Check

I/O       Requirements

│

▼

I/O-Mapped I/O

Justification

The algorithm makes the I/O mapping decision dynamically according to the given latency and

bandwidth requirements, as required by the question.

- Design	pseudocode	for	a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

Bus Arbitration Algorithm

This question asks for pseudocode to manage multiple devices communicating through PCIe,

USB, and Thunderbolt interfaces.

Aim

To design a bus arbitration algorithm that provides fair access to the communication bus for

multiple devices.

Input

- List of connected devices

- Interface type: PCIe, USB, or Thunderbolt

- Bus access requests

Output

- Bus allocated to the selected device

- Fair communication among devices

Pseudocode

BEGIN

Initialize Device List

Initialize Bus Arbitration

WHILE System is Running

Check all devices for bus requests

IF a device requests the bus THEN

Add device to Request Queue

Select device using Round-Robin method

Grant Bus Access to selected device

IF selected device uses PCIe THEN

Transfer data through PCIe

ELSE IF selected device uses USB THEN

Transfer data through USB

ELSE IF selected device uses Thunderbolt THEN

Transfer data through Thunderbolt

END IF

Release Bus

Move to next device in the queue

ELSE

Continue monitoring devices

END IF

END WHILE

END

Working

- The system continuously checks all connected devices for bus requests.

- When a device requests access, it is added to the request queue.

- The round-robin method selects devices in a fair sequence.

- Bus access is granted to the selected device.

- Data is transferred using its respective interface:

  - PCIe → PCIe communication

  - USB → USB communication

  - Thunderbolt → Thunderbolt communication

- After the transfer, the device releases the bus.

- The next requesting device gets an opportunity.

Example

Request Queue:

Device A → PCIe

Device B → USB

Device C → Thunderbolt

Device D → USB

Round-Robin:

A → B → C → D → A → B → ...

Justification

The round-robin arbitration provides fair access to the bus and prevents one device from

continuously occupying the communication resource. The algorithm supports devices using PCIe,

USB, and Thunderbolt as required.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Hybrid I/O Communication Mechanism

This question asks for pseudocode that uses polling for low-speed devices and DMA with

interrupts for high-speed devices.

Aim

To design a hybrid I/O mechanism that selects polling or DMA according to the speed of the device.

Input

- Device type

- Device speed

- Data size

Output

- Polling used for low-speed devices

- DMA with interrupts used for high-speed devices

Pseudocode

BEGIN

Initialize I/O Devices

WHILE System is Running

Check Device

IF Device Speed = LOW THEN

Start Polling

WHILE Data is Not Ready

Check Device Status

END WHILE

Read Data from Device

Store Data in Memory

ELSE IF Device Speed = HIGH THEN

Initialize DMA Controller

Set Source = High-Speed Device

Set Destination = Memory

Set Transfer Size = Data Size

Start DMA Transfer

CPU performs other tasks

WAIT for DMA Interrupt

IF DMA Transfer is Successful THEN

Process Data in Memory

ELSE

Display "DMA Transfer Failed"

END IF

END IF

END WHILE

END

Working

- The system checks the device speed.

- For a low-speed device, the CPU uses polling to repeatedly check whether data is ready.

- Once data becomes available, the CPU reads and stores it in memory.

- For a high-speed device, the DMA controller is initialized.

- DMA transfers data directly from the device to memory.

- The CPU can perform other tasks during the DMA transfer.

- After the transfer is completed, DMA generates an interrupt.

- The CPU handles the interrupt and processes the received data.

Decision Flow

Start

│

▼

Check Device Speed

│

┌──────┴──────┐

│             │

LOW           HIGH

│             │

▼             ▼

Polling         DMA

│             │

▼             ▼

Read Data     Transfer to Memory

│             │

│             ▼

│       DMA Interrupt

│             │

└──────┬──────┘

▼

Process Data

│

▼

End

Justification

The hybrid mechanism uses polling for low-speed devices, where continuous checking has

relatively low overhead, and DMA with interrupts for high-speed devices, where direct data

transfer reduces CPU involvement. This matches the requirement specified in the assessment.

## MARKS ALLOCATION

|  | Problem Understanding (20%) | Algorithm Design (30%) | Use of Control Structures (20%) | Pseudocode Structure (10%) | Completeness (20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

## RUBRICS FOR CALCULATION:

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |