NAME: Prathiba .K

REG NO: 192421163

COURSE: COMPUTER ARCHITECTURE

CODE: CSA1204

Q1. Design and Simulation of Register Transfer, Arithmetic and Logic Operations

Aim:
To design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR, and to analyze the role of the ALU during processing.

1. Introduction

A register is a small, high-speed storage location inside the processor used to temporarily store data and instructions. Register Transfer refers to moving data from one register to another.

The Arithmetic Logic Unit (ALU) is the main processing unit responsible for performing arithmetic and logical operations. In this experiment, multiple registers are connected to an ALU to demonstrate how data is transferred and processed.

2. Basic Components

The system consists of:

- Registers – Store temporary data.

- Bus – Transfers data between registers and the ALU.

- ALU – Performs arithmetic and logical operations.

- Control Unit – Generates control signals to select the required operation.

- Output Register – Stores the final result.

3. Block Diagram

┌─────────────┐

│  Register A │

└──────┬──────┘

│

▼

┌───────────┐

│           │

│    ALU    │──────► Output Register

│           │

└─────▲─────┘

│

│

┌──────┴──────┐

│  Register B │

└─────────────┘

The registers provide input data to the ALU. The ALU performs the selected operation and sends the result to the output register.

4. Register Transfer Operations

Register transfer can be represented using RTL notation.

For example:

R1 → R2

This means that the contents of register R1 are transferred to R2.

Other examples:

R1 → R2

R2 → R3

R3 → R4

The transfer takes place when the corresponding control signal is activated.

5. Arithmetic Operations

Assume:

R1 = 10

R2 = 5

Addition

R3 ← R1 + R2

Therefore,

R3 = 10 + 5 = 15

Subtraction

R3 ← R1 - R2

Therefore,

R3 = 10 - 5 = 5

6. Logical Operations

The ALU can also perform logical operations on binary data.

AND Operation

R3 ← R1 AND R2

Example:

R1 = 1010

R2 = 1100

R3 = 1000

OR Operation

R3 ← R1 OR R2

Example:

R1 = 1010

R2 = 1100

R3 = 1110

7. Sequence of Operations

A possible execution sequence is:

| Step | Operation | Result |
| --- | --- | --- |
| 1 | Load data into R1 | R1 = 10 |
| 2 | Load data into R2 | R2 = 5 |
| 3 | Transfer R1 to ALU | First operand selected |
| 4 | Transfer R2 to ALU | Second operand selected |
| 5 | Select ADD operation | R3 = 15 |
| 6 | Select SUB operation | R3 = 5 |
| 7 | Select AND operation | Logical result obtained |
| 8 | Select OR operation | Logical result obtained |

8. Role of ALU

The ALU plays an important role in processing data. It receives operands from registers, performs the operation specified by the control unit, and sends the result to the destination register.

Thus, the ALU acts as the central processing element for both arithmetic and logical operations.

9. Result and Analysis

The designed system successfully demonstrates register-to-register data transfer and ALU-based processing. Addition and subtraction are performed as arithmetic operations, while AND and OR are performed as logical operations.

The experiment shows that proper control signals are required to select registers, transfer data and determine the ALU operation. Multiple registers allow intermediate values to be stored and transferred efficiently.

10. Conclusion

The register transfer system was successfully designed and simulated. The experiment demonstrated how data moves between registers and how the ALU performs arithmetic and logical operations. It also helped in understanding the sequence of operations and the important role of the ALU in processor data processing.

Q2. Single-Bus and Multi-Bus Organization

Aim

To create a model of a computer system using single-bus and multi-bus organization, perform data transfer between registers, memory and I/O, and compare their time and efficiency.

1. Introduction

A bus is a communication pathway used to transfer data between different components of a computer system. Registers, memory and I/O devices communicate with each other through buses.

Two common bus organizations are:

- Single-bus organization

- Multi-bus organization

2. Single-Bus Organization

In a single-bus system, all registers and components share one common bus.

R1 ──┐

R2 ──┤

R3 ──┤──► Common Bus ──► ALU

R4 ──┘                    │

▼

Memory/I/O

Only one data transfer can normally use the common bus at a time. Therefore, multiple operations may need to wait for the bus.

3. Multi-Bus Organization

A multi-bus system uses two or more buses simultaneously.

┌──── Bus A ────┐

R1 ────┤               │

│      ALU      ├──► Result

R2 ────┤               │

└──── Bus B ────┘

Multiple operands can be transferred at the same time, reducing the number of clock cycles required.

4. Data Transfer

Example:

R1 = 20

R2 = 10

For addition:

R3 ← R1 + R2

In a single-bus organization, the operands may need to be transferred sequentially.

In a multi-bus organization, both operands can be supplied to the ALU simultaneously.

5. Comparison

| Feature | Single Bus | Multi Bus |
| --- | --- | --- |
| Number of buses | One | Multiple |
| Data transfer | Sequential | More parallel |
| Hardware complexity | Low | High |
| Cost | Lower | Higher |
| Speed | Comparatively lower | Higher |
| Bottleneck | More likely | Reduced |
| Efficiency | Moderate | High |

6. Analysis

The single-bus organization is simple and economical but can become a bottleneck because different components compete for the same bus.

The multi-bus organization provides greater parallelism. Multiple data transfers can occur simultaneously, reducing waiting time and improving processor performance. This directly addresses the assessment requirement to analyze how multiple buses improve parallel transfer and reduce bottlenecks.

Result

The single-bus and multi-bus systems were modeled successfully. The multi-bus organization provides faster and more efficient data transfer because it supports greater parallelism.

Conclusion

Multi-bus organization offers better performance than single-bus organization, although it requires additional hardware and increases system complexity.

Q3. Five-Stage Instruction Pipeline

Aim

To simulate a 5-stage instruction pipeline consisting of Fetch, Decode, Execute, Memory and Write-back, and compare its performance with a non-pipelined processor using speedup and throughput.

1. Introduction

Pipelining is a technique used in processors to improve instruction execution speed. An instruction is divided into several stages, and different instructions can occupy different stages simultaneously.

The five stages are:

- IF – Instruction Fetch

- ID – Instruction Decode

- EX – Execute

- MEM – Memory Access

- WB – Write Back

2. Pipeline Structure

Instruction

│

▼

┌─────────┐

│   IF    │ Instruction Fetch

└────┬────┘

▼

┌─────────┐

│   ID    │ Instruction Decode

└────┬────┘

▼

┌─────────┐

│   EX    │ Execute

└────┬────┘

▼

┌─────────┐

│   MEM   │ Memory Access

└────┬────┘

▼

┌─────────┐

│   WB    │ Write Back

└─────────┘

3. Pipeline Execution

Consider five instructions:

I1

I2

I3

I4

I5

The execution can be represented as:

| Cycle | I1 | I2 | I3 | I4 | I5 |
| --- | --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |  |
| 2 | ID | IF |  |  |  |
| 3 | EX | ID | IF |  |  |
| 4 | MEM | EX | ID | IF |  |
| 5 | WB | MEM | EX | ID | IF |
| 6 |  | WB | MEM | EX | ID |
| 7 |  |  | WB | MEM | EX |
| 8 |  |  |  | WB | MEM |
| 9 |  |  |  |  | WB |

After the pipeline is filled, one instruction can complete during each cycle.

4. Non-Pipelined Execution

In a non-pipelined processor, an instruction completes all five stages before the next instruction starts.

For five instructions:

5 × 5 = 25 stage cycles

With an ideal five-stage pipeline:

5 + (5 - 1) = 9 cycles

5. Speedup

Speedup is calculated as:

Speedup = Non-pipelined execution time / Pipelined execution time

For the above example:

Speedup = 25 / 9 ≈ 2.78

The actual speedup depends on the number of instructions and pipeline delays.

6. Throughput

Throughput represents the number of instructions completed per unit of time.

After the pipeline is filled, the processor can ideally complete approximately one instruction per clock cycle.

Result

The five-stage pipeline was successfully simulated. The pipelined processor required fewer cycles than the equivalent non-pipelined processor.

Conclusion

Pipelining improves processor performance by allowing different instructions to be processed simultaneously in different stages. It increases instruction throughput and reduces the average time required per instruction.

Q4. Pipeline Hazards and Hazard Resolution Techniques

Aim

To develop a pipeline execution scenario containing data, control and structural hazards, observe their effects and implement stalling, data forwarding and branch prediction techniques.

1. Introduction

A pipeline hazard is a situation that prevents the next instruction from executing in its normal pipeline cycle.

There are three major types:

- Data Hazard

- Control Hazard

- Structural Hazard

2. Data Hazard

A data hazard occurs when one instruction depends on the result produced by an earlier instruction.

Example:

I1: R1 ← R2 + R3

I2: R4 ← R1 + R5

Here, I2 requires R1, which is produced by I1.

Solution: Data Forwarding

The result of I1 can be directly forwarded from the pipeline to I2 without waiting for the value to be written back into the register.

I1 → EX → Forwarding → I2

This reduces unnecessary stalls.

3. Control Hazard

A control hazard occurs because of branch or jump instructions.

Example:

BEQ R1, R2, TARGET

The processor may not immediately know which instruction should be fetched next.

Solution: Branch Prediction

The processor predicts whether the branch will be taken or not taken.

If the prediction is correct, execution continues without a major delay.

4. Structural Hazard

A structural hazard occurs when two instructions require the same hardware resource at the same time.

For example, if instruction fetch and data memory access require the same memory resource simultaneously, a conflict can occur.

Solution: Stalling

The processor temporarily stops one instruction until the required resource becomes available.

Instruction → Execute

↓

STALL

↓

Continue

5. Hazard Comparison

| Hazard | Cause | Example | Solution |
| --- | --- | --- | --- |
| Data | Data dependency | R1 used by next instruction | Forwarding |
| Control | Branch/jump | BEQ | Branch prediction |
| Structural | Resource conflict | Same hardware resource | Stalling |

6. Effect on Performance

Hazards can introduce additional clock cycles called pipeline stalls. These reduce the effective speedup and throughput of the processor.

Techniques such as forwarding, branch prediction and proper resource allocation reduce the number of stalls and improve pipeline performance.

Result

The three major pipeline hazards were identified and their effects on instruction execution were analyzed. Stalling, forwarding and branch prediction were applied as appropriate hazard-resolution techniques.

Conclusion

Pipeline hazards can reduce processor performance, but suitable techniques can minimize their impact. Data forwarding handles data dependencies, branch prediction reduces control-hazard penalties, and stalling handles situations where resources are unavailable.

Q5. Superscalar, Out-of-Order, Speculative Execution and SMT

Aim

To analyze a processor supporting superscalar, out-of-order and speculative execution, and study hyper-threading and simultaneous multithreading (SMT) to evaluate instruction throughput and resource utilization.

1. Introduction

A superscalar processor can execute more than one instruction in a single clock cycle by using multiple execution units.

For example:

┌──► ALU 1

Instructions ─┼──► ALU 2

├──► FPU

└──► Load/Store Unit

This allows independent instructions to execute in parallel.

2. Superscalar Execution

Suppose four independent instructions are available:

I1 → ADD

I2 → SUB

I3 → AND

I4 → OR

A superscalar processor with multiple execution units may execute several of them simultaneously.

This increases instruction throughput.

3. Out-of-Order Execution

In-order execution processes instructions according to their program order.

Out-of-order execution allows an instruction that is ready to execute before an earlier instruction that is waiting for data or a resource.

Example:

I1 → Waiting for data

I2 → Ready

I3 → Ready

Instead of keeping the processor idle, I2 and I3 can execute while I1 is waiting.

This improves resource utilization.

4. Speculative Execution

Speculative execution allows the processor to execute instructions before it is certain that they are required.

This is particularly useful around branch instructions.

Branch

│

├── Path A → Speculative execution

│

└── Path B → Alternative path

If the prediction is correct, useful work has already been completed. If it is incorrect, the speculative results are discarded and the correct path is executed.

5. Hyper-Threading and SMT

Simultaneous Multithreading (SMT) allows multiple instruction streams or threads to share processor resources.

Hyper-threading is a form of SMT technology that allows a physical processor core to appear as multiple logical processors.

Example:

Physical Core

│

┌───────┴───────┐

▼               ▼

Thread 1         Thread 2

│               │

└───────┬───────┘

▼

Shared Resources

This can improve processor utilization when one thread cannot fully use all available resources.

6. Performance Comparison

| Technique | Main Advantage |
| --- | --- |
| Superscalar | Executes multiple instructions per cycle |
| Out-of-order | Executes ready instructions without waiting unnecessarily |
| Speculative execution | Performs useful work before branch outcome is confirmed |
| SMT | Allows multiple threads to share processor resources |
| Hyper-threading | Improves utilization of a physical processor core |

7. Instruction Throughput

Instruction throughput refers to the number of instructions completed per unit of time.

A superscalar processor can achieve higher throughput than a scalar processor because several instructions may execute in parallel.

Out-of-order execution further improves utilization by keeping available execution units busy.

8. Resource Utilization

The major processor resources include:

- ALUs

- Floating-point units

- Load/store units

- Registers

- Instruction issue units

Superscalar and SMT techniques attempt to keep these resources active rather than leaving them idle.

Result

The operation of superscalar, out-of-order and speculative execution was analyzed along with hyper-threading and SMT. These techniques allow multiple instructions or threads to make progress in parallel and can improve instruction throughput and processor resource utilization.

Conclusion

Superscalar execution increases parallel instruction execution, out-of-order execution reduces waiting for dependent resources, speculative execution improves performance around branches, and SMT improves utilization by allowing multiple threads to share processor resources. Together, these techniques contribute to higher processor performance.